# Accuracy, Precision, and Tolerance

module_20 | Grade 8-12 | 5 days

# Vocabularycards

## accuracy

### Definition

The degree to which a measurement or calculation agrees with the true value.

### Example

If a thermometer reads 100°C when the actual temperature is 100°C, it is considered accurate.

### Visualdescription

A target with an arrow hitting the bullseye, indicating closeness to the true value.

## precision

### Definition

The degree to which repeated measurements under unchanged conditions show the same results.

### Example

If a scale repeatedly shows a weight of 50 kg when weighing the same object, it demonstrates high precision.

### Visualdescription

A target with arrows clustered closely together, indicating consistency in measurements.

## resolution

### Definition

The smallest increment that can be measured by a measurement system.

### Example

A digital scale that measures in increments of 0.01 kg has a resolution of 0.01 kg.

### Visualdescription

A ruler showing fine divisions, illustrating the smallest measurable unit.

## repeatability

### Definition

The ability to obtain the same measurement under the same conditions over multiple trials.

### Example

A stopwatch recording the same time for a race multiple times demonstrates repeatability.

### Visualdescription

A series of identical measurement results displayed in a graph or chart.

## uncertainty

### Definition

A quantifiable indication of doubt about the measurement result.

### Example

A measurement of 10.0 ± 0.2 cm indicates an uncertainty of 0.2 cm.

### Visualdescription

A measurement tape with a highlighted area showing the range of uncertainty.

## tolerance

### Definition

The acceptable range of variation in a measurement or value.

### Example

A tolerance of ± 5% means that the actual measurement can vary by 5% above or below the stated value.

### Visualdescription

A gauge showing a range with upper and lower limits marked clearly.

## absolute error

### Definition

The difference between the measured value and the true value.

### Example

If the true length of an object is 15 cm and it is measured as 14.5 cm, the absolute error is 0.5 cm.

### Visualdescription

A diagram showing a measured value and the true value with the difference highlighted.

## percent error

### Definition

The absolute error expressed as a percentage of the true value.

### Example

If the true value is 50 and the measured value is 48, the percent error is (2/50) * 100 = 4%.

### Visualdescription

A pie chart illustrating the proportion of error relative to the whole.

## reference value

### Definition

A known or accepted value used as a comparison for measurements.

### Example

The melting point of pure water at 0°C is a reference value for temperature measurements.

### Visualdescription

A thermometer with a highlighted point indicating the reference value.

## acceptance criterion

### Definition

The conditions that must be met for a measurement or result to be considered acceptable.

### Example

An acceptance criterion may state that the measurement must fall within ± 2% of the reference value.

### Visualdescription

A checklist with ticks next to criteria that have been met.